# Lab 11

# Surface Analysis

Surface analyses are a powerful tool in GIS environments. The surface toolset in ArcGIS Pro allows the user to quantify and visualize landform terrain and dynamics that are represented within a Digital Elevation Model (DEM). Beginning with a raster DEM, the surface toolset in ArcGIS Pro allows users to gain information by producing a new dataset from the original input DEM. Products such as contours, angles, slopes and shaded relief can be calculated using the surface toolset.

In this lab we will explore raster surfaces and raster surface analyses. A 10-meter resolution DEM of Chester County, Pennsylvania located in Southeastern Pennsylvania is supplied. From this DEM, four new surfaces will be created: slope, aspect, hillshade and a contour map. Based on these derived raster surfaces and road data a suitability analysis for a hypothetical building site will be performed. This lab will rely heavily on the Spatial Analyst toolset and will go over a simple raster addition in order to determine suitable building locations based on criteria given in the instructions. General instructions are provided. In order to complete the lab online help resources should be used (ArcGIS help). Deliverables are what will need to be turned in for credit, and are outlined following the instructions.

Key Search Terms: Slope, Aspect, Spatial Analyst, Contour, Reclassify, Raster Calculator, Polygon to Raster, Select by attribute, Raster to Polygon, Clip

Instructions:

- In ArcGIS Pro insert a new basemap “Light Gray Canvas Basemap”.

- Load the Digital Elevation model titled “DEM_10m.tif” into your map document.

- Create three different raster maps showing slope, aspect and hillshade. Additionally, create a contour map. (Hint: Each of the four tools can be found under Spatial Analyst within the Surface toolbox.)

  - Inputs for the Slope tool: Scaling = Degree, Z-Factor = 1

  - Inputs for the Aspect tool: Accept all defaults

  - Inputs for the Hillshade tool: Azimuth = 315, Altitude = 45, Z-Factor = 1 
(Azimuth is the angle of the sun, which means that in this scenario the sun is coming from the northwest and is at an altitude of 45 degrees above the horizon. This is mid-late afternoon sun in the study area.)

  - Inputs for the Contour tool: Contour Interval = 20, Base Contour = 0, Z-Factor = 1

- Now, using the reclassify tool, find a suitable area to build a home that adhere to the builder’s criteria: South facing (aspect between 150-210°), Slope <10 °, Hillshade > 150, and the elevation should be between 320 meters – 460 meters above sea level.

  - Use the reclassify tool in the spatial analysis toolbox with inputs of your aspect, slope and hillshade rasters (you will will need to run this tool three times, once for slope, once for hillshade and once for aspect). When using the reclassify tool, make Boolean suitability rasters where a value of 1 matches the builder’s criteria and a value of 0 means the location is not suitable.

- Because the contour map is not a raster, we cannot simply reclassify based on elevation in one step.

  - To add elevation from contours to our analysis, we must convert the contour map to a raster data set. To do this, use the “Polyline to Raster” tool.

Input features= contour layer, Value field= “Contour”, Cell assignment

type = “Maximum length”, Priority field= “none”, Cellsize= “800”.

  - Now that the contours have been converted to a raster, reclassify the elevations between 320-460 m above sea level to 1 and areas above 460 m and below 320 m as 0.

- Now that you have four suitability rasters created, use the raster calculator to add the suitability rasters together. Because suitability values were set to 1 in each of the four suitability rasters created, areas with a value of 4 are suitable sites.

  - Raster calculator is located in: Tools>Spatial Analyst>raster Calculator. To perform the raster calculation double click the first layer that will be in the calculation and then double click the “+” operator. Do this sequentially for the next four layers involved in the calculation and then click “run”. Make sure to specify where the output of the calculation will be saved.

  - Example: Reclass_Slope + Reclass_Aspect + Reclass_Hill + Reclass_Elev

- For your final suitability map, use the “Go to XY” tool under the Map tab to find the following location: 39.9673837, -75.5815914

  - Set the scale to 1:30,000.

  - Create a map showing areas that are suitable to live.

Deliverables:

- What is a Z-factor? If your ground units are in meters and the output elevation needs to be in feet, what would the correct Z-factor be?

- Briefly explain the differences between the hillshade map with a Z-Factor of 1 and a Z-factor of 0.3048.

- In the contour dialog, what is the base contour describing? What is a contour interval?

- Maps (not screenshots) with scale bars, titles, north arrows, proper symbology and basemap of:

  - Slope

  - Aspect

  - Hillshade

  - Final Suitability Locations with inset map