# Lab 8

# Advanced Cartography

The goal of a map is to quickly and efficiently convey a message to an audience in an aesthetically pleasing format.  Anyone can create a map, but only a few can produce a map that speaks clearly to its audience. The creation of a map is as much art as it is technical.  The cartographic process goes much further than putting together the data and general map layout. Steps often included in this process are reducing data complexity, editing symbology, labeling clean up, and removing extraneous elements that might cause confusion.

This project aims to display a large amount of information within a single map using selective labeling, annotations, and an inset. The purpose of the map will be to display general lodging information and recreational trails located near Bryce Canyon National park, while also providing general directions on how to get to the park using an inset map. General instructions are provided. In order to complete the lab online help resources should be used (ArcGIS help). Deliverables are what will need to be turned in for credit, and are outlined following the instructions.

Key Search Terms: Make XY Event Layer, Definition Query, Symbology, Convert Labels to Annotations, Inset, Extent Indicator

Instructions:

- Open ArcGIS Pro and create a map titled Lab8Main; load the provided geographic data near Bryce Canyon and a table of lodging information. Right click on the map to set the reference scale (1:200,000) and display the town of Bryce, Bryce Canyon National Park’s boundary, and the neighboring Tropic Valley. Change the basemap to Terrain with Labels and then turn off the World Terrain Reference in the table of contents for now.

- Display the x and y coordinates of the table provided representing lodging and campgrounds near the park (Right click on the table in the Table of Contents and select Display XY Data to add the points to your map.)

  - Symbolize this point feature so it is legible without the need to reference a legend (use ArcGIS symbology within the Gallery tab for Campground and Hotel symbols).

  - Label only the campground names using an SQL query in the Labeling Gallery (Labeling tab).

    - To label only campgrounds, select the Labeling Gallery and create an SQL query on the Class_1 where CATEGORY is Equal to Campground. Use about a size 8 for these labels. Make sure the “Label” option is clicked on the far left of the bar.

- Load trails feature and create a thin, brown, dashed line symbology. Make them slightly transparent.

  - Use the Dashed 2:2 symbology with a brown color and 0.5 width; set the transparency to about 35%.

- For the roads, create a symbology to differentiate the road type between paved and backcountry roads.

  - Use the “Condition” field in the attribute table to differentiate between “Paved” and “Backcountry” roads.

  - Label only Highway 12 and Main Park Road based using of the FULLNAME field and using an SQL Query for those two features (see Step 2 for SQL query example).

- Finally, add the land ownership layer and customize their symbology using the following guidelines for land ownership: 
National Park Service – Purple, BLM – Yellow, U.S. Forest Service – Green, State lands – Light blue, private lands – No color (additionally, remove borders from this symbology). Adjust the transparency of the land ownership layer to allow for the underlying terrain basemap being visible.

  - For each admin designation select the above color and remove the boundary line.

  - Remove “all other values”

  - Adjust transparency on the Appearance Gallery to about 40%.

- Label the National Park and neighboring National Forests and National Monument (Hint: NPS, USFS, BLM).

  - Label the feature based on the LABEL_FEDERAL field.

- Once all features are labeled, run the “Convert Labels to Annotations” tool on the map. This will create a new geodatabase feature that stores all text elements separate from your map features.

  - Analysis Gallery -> Tools -> Convert Labels to Annotations.

  - New annotation feature will be automatically added to map.

  - Click on the new annotation group and select Move within the Edit Gallery.

  - Click on the desired annotation and move to a location using the yellow selection icon. When finished, save your edits with the Save option.

- Create a 8.5”x11” landscape layout map and insert your Lab8Main map; add the title, legend, north arrow, and scale bar elements to the right side of the map.

  - Insert Gallery -> Text (Title), North Arrow, Scale Bar, and Legend.

    - To edit these elements click on them in the layout and utilize the format and design Galleries at the top of the page.  On the scale bar, the Format tab allows for the changing of colors while the Design tab edits the number of division and unit and number placements.

    - Modify any black parts of the elements to be a Grey 70%.

    - Additionally, modify the scale bar to have only a single division and two subdivisions and so that all text and numbers fall beneath the scale bar, rather than above or to the side.

  - While on the layout page, Insert -> Dynamic Text -> Service Layer Credits.

    - Turn this dynamic text off in the Table of Contents to remove the basemap text.

  - A neat line can be added using the Insert Gallery -> Rectangle. Modify to be hollow and with a width and color border matching the main map.

- To create an inset map, create a new map titled Lab8Inset and zoom to the southern half of Utah using the Light Grey Canvas Base map, turning off the reference element.

  - Add the feature “Municipalities” inset and label the two cities.

  - Add an online feature using the “All Portal” and search for “USA Major Highways”. Once your labels are complete, convert the labels to annotations to place the road and city labels as desired.

  - Finally, add a purple polygon displaying the location of Bryce Canyon National Park using a definition query on the Land Ownership feature.

- Insert this map as an inset map within the Layout Map approximately 3.25”x2.25” in size in a location that minimizes obstruction of any features in the main map.

  - Add an extent indicator to the inset map to show the location of the main map within it. Be sure I-15 and St. George are visible.

    - Insert Gallery -> Map Frame -> Lab 8 Inset.

Deliverables:

- A layout map neatly displaying Bryce Canyon National Park and the surrounding areas that includes an inset map showing major roads and the parks location in Southern Utah. Be sure to pay special attention to elements including labels, map elements, and the inset map placement.