# Lab 06

# Creating & Editing Features, Geodatabases, Metadata

Geodatabases work as the foundation for storing geographic data in the ArcGIS platform. Being able to create new data sets is vital when data does not exist digitally or is lacking in detail. During this lab you will learn how to work with geodatabases. General instructions are provided as a guide. General instructions are provided. In order to complete the lab online help resources should be used (ArcGIS help). Deliverables are what will need to be turned in for credit, and are outlined following the instructions.

Key Search Terms: Geodatabases, Edit, Digitize, Feature Class, Line Feature, Point Feature, Modify, Attribute Field, Save edits, Metadata

Instructions:

- Create a new map for the Great Salt Lake. Change the basemap to “Imagery”. As we will not be adding existing data, manually change the coordinate system to NAD 1983 UTM Zone 12N.

- Create a new polygon feature in your geodatabase titled Great_Salt_Lake_Boundary. Be sure the coordinate system is NAD 1983 UTM Zone 12N for this and all features created.

- Digitize the boundary of the Great Salt Lake as a polygon at 1:750,000 scale.

  - Remove the four largest interior islands in the southern section of the lake. This can be accomplished by digitizing them into the lake feature and then clipping and discarding the areas that overlap them. More information can be found at:

http://pro.arcgis.com/en/pro-app/help/editing/cut-a-hole-in-a-polygon-feature.htm#GUID-3D8CBD47-1BA2-4419-9A84-D196C2EB5A0A

  - Save edits before editing attribute table

  - In the Great Salt Lake feature class attribute table create a data source attribute and populate with “ArcGIS Imagery Basemap”.

- Create a new line feature and digitize the three major rivers that feed the Great Salt Lake. Digitize these at a 1:100,000 scale and at least 5 miles away from lake.

  - Create river name and data source attribute fields and populate.

- Finally, create a point feature and digitize the highest point of the four islands outlined in the lake earlier. Digitize these at a 1:20,000 scale using the Topographic Basemap.

  - Create a height, island, and source attribute field and populate.

- Update each feature’s metadata to include information for Tags, Summary, Description, and Credits for all three features.

- Create a layout map showing the lake, rivers, and highest elevation points over the Terrain basemap without the Reference Layer active. Label the 4 points based on their recorded elevation as well as the rivers.

Deliverables:

- Export a layout map showing your digitized features point, line, and polygon features set at 1:600,000 scale.

- The lab 06 geodatabase with three features, each with proper geometry, attributes coordinate system, and metadata. Make sure to zip the geodatabase file.

- A screenshot of the metadata tab for the Great_Salt_Lake_Boundary.